Micro Flapper Wing Flyer

Power Distribution Design Decisions

November 08, 2025

Adam Polanski

101225888

# Capacitors

Electrolytic Capacitors

Electrolytic capacitors are designed with high capacitance values, typically ranging from 1 µF to several thousand µF, making them ideal for energy storage and smoothing out low-frequency voltage ripple in power supply circuits. Because of their large capacity, they are effective at reducing voltage dips and maintaining a stable DC voltage under varying loads. However, electrolytic capacitors are polarized components, meaning they must be connected with the correct polarity to prevent damage or failure. They also have a higher equivalent series resistance (ESR) compared to other capacitor types, which makes them less effective at filtering high-frequency noise or responding to rapid voltage fluctuations. In electronic circuits, electrolytic capacitors are most commonly used for bulk decoupling, power supply filtering, and energy buffering, where they excel at handling large, slow voltage changes that occur during system start up or heavy load transitions.

Ceramic Capacitors

Ceramic capacitors have relatively low capacitance values, typically ranging from a few picofarads (pF) to several microfarads (µF), but they offer very fast response times due to their low internal resistance and inductance. Unlike electrolytic capacitors, ceramics are non-polarized, meaning they can be connected in any direction without risk of damage. They feature low equivalent series resistance (ESR) and low equivalent series inductance (ESL), making them highly effective for high-frequency noise filtering and for stabilizing voltage regulators or buck converters by reducing high-speed ripple. Because of their ability to react quickly to fast voltage transients, ceramic capacitors are commonly placed close to integrated circuit (IC) power pins, such as on microcontrollers or switching regulators, to filter out switching noise and maintain clean power delivery. Overall, they excel at handling small, fast voltage changes and are essential for ensuring stable, noise-free operation in high-speed electronic circuits.

Power Distribution Schematic

Right after the 12 V power enters the circuit, a 470 µF electrolytic capacitor (rated above 12 V, ideally 25 V) is connected between the +12 V line and ground. This capacitor provides bulk energy storage, helping to supply the buck converters during large current surges and prevent voltage droop on the 12 V rail.

In parallel with each buck converter’s input, a 4.7 µF ceramic capacitor (rated above 12 V) is placed across the VIN pins and ground to suppress high-frequency switching spikes that the larger electrolytic capacitor cannot effectively filter.

At the output of each buck converter, a 10 µF ceramic capacitor (rated for at least 10 V) is installed between the VOUT pins and ground to smooth out switching ripple and ensure a stable DC output for sensitive components.

On the power input lines of the L293D H-Bridge, a 0.1 µF ceramic capacitor (rated above 12 V) should be placed as close as possible to the H-Bridge power pins, filtering out any small, high-frequency voltage fluctuations before they reach the IC.

Across the motor terminals, a 0.1 µF ceramic capacitor is connected in parallel with the motor leads to suppress electromagnetic interference (EMI) and reduce electrical noise generated by the motors/coils/magnets.